HARRIS TOOL COMPANY MODEL NO. SJW-4 40 X 54 FRONT LOAD SUPER DUTY WASHER WITH FOLLOWING MACHINE SPECIFICATIONS:

- TOGGLE SWITCH OPERATED STAINLESS DISC TYPE OIL SKIMMER

- 18KW ELECTRIC HEATER

- TANK DEBRIS SCREEN (750 SQ. IN.)

- 7.5 HP CAST IRON VERTICAL SEALESS PUMP W/SCREEN(150 GPM @ 55 PSIG)

- 60 MIN.SPRING WOUND WASH TIMER

- THERMOSTAT SWITCH W/ 175 DEGREES F SETTING

- 37 SPRAY JET NOZZLES

- 243 GALLON CLEANING SOLUTION TANK

- LOW WATER SHUT-OFF SWITCH

- 2000 LB. LOAD CAPACITY

- 54” INSIDE HEIGHT

- 1/20 HP GEARMOTOR/GEAR DRIVEN 40” DIA. ROLL-OUT TURNTABLE W/ REMOVEABLE PARTS RING

- 240/480/60/3 ELECTRICAL REQUIREMENTS(PLEASE SPECIFY VOLTAGE)

- INTERMEDIATE SPRAY BAR W/ NOZZLES

- ½” PIN/KNUCKLE SEALESS HINGED DOOR W/ HANDLE, LATCH, & SAFETY SWITCH.

- 10 GAUGE TANK/12 GAUGE DOOR & CABINET STEEL PLATE MATERIAL

- 6” DIA. CABINET VENT

- 74.5” OVERALL HEIGHT

- 47”W X 86”D FLOOR SPACE REQUIRED W/ DOOR CLOSED

- DURABLE BLUE POWDER COAT FINISH

- 2000 LBS. SHIPPING WT.