HARRIS TOOL COMPANY MODEL NO. WA-JUMBO 50 X 72 FRONT LOAD SUPER DUTY WASHER WITH FOLLOWING MACHINE SPECIFICATIONS:

- TOGGLE SWITCH OPERATED STAINLESS DISC TYPE OIL SKIMMER

- 18KW ELECTRIC HEATER

- TANK DEBRIS SCREEN (264 SQ. IN.)

- 10 HP CAST IRON VERTICAL SEALESS PUMP W/SCREEN(240 GPM @ 55 PSIG)

- 60 MIN.SPRING WOUND WASH TIMER

- 175 DEGREES F THERMOSTAT SWITCH SETTING

- 33 SPRAY JET NOZZLES

- 300 GALLON CLEANING SOLUTION TANK

- LOW WATER SHUT-OFF SWITCH

- 3,500 LB. LOAD CAPACITY

- 72” INSIDE HEIGHT

- 1/15 HP GEARMOTOR/GEAR DRIVEN 50” DIA.TURNTABLE W/ REMOVEABLE PARTS RING & JOG SWITCH

- 240/480/60/3 ELECTRICAL REQUIREMENTS(PLEASE SPECIFY VOLTAGE)

- INTERMEDIATE SPRAY BAR W/ NOZZLES

- ½” PIN/KNUCKLE HINGED DOOR W/ HANDLE, LATCH, AND SAFETY SWITCH

- 10 GAUGE TANK/12 GAUGE DOOR & CABINET STEEL PLATE MATERIAL

- 106” OVERALL HEIGHT

- 66”W X 72”D FLOOR SPACE REQUIRED W/ DOOR CLOSED

- PRECAUTION BLUE PAINT

- 2600 LBS. SHIPPING WT.