HARRIS TOOL COMPANY MODEL NO. WA-JUMBO-HD

50 X 72 FRONT LOAD SUPER HEAVY DUTY WASHER WITH FOLLOWING MACHINE SPECIFICATIONS:

- TOGGLE SWITCH OPERATED STAINLESS DISC TYPE OIL SKIMMER

- 27 KW ELECTRIC HEATER

- REMOVEABLE TANK DEBRIS SCREEN (330 SQ. IN.)

- 15 HP CAST IRON VERTICAL SEALESS PUMP W/SCREEN(315 GPM @ 55 PSIG)

- 60 MIN.SPRING WOUND WASH TIMER

- 175 DEGREES F PROCESS THERMOSTAT CONTROLLER

- CRANE LOAD SLOT

- FORK LIFT CHANNELS FOR PORTABILITY

- 50 REPLACEABLE/CLEANABLE SPRAY JET NOZZLES

- 300 GALLON CLEANING SOLUTION TANK

- LOW WATER SHUT-OFF SWITCH

- 5,000 LB. LOAD CAPACITY

- 72” INSIDE HEIGHT

- 1/2 HP GEARMOTOR/GEAR DRIVEN 50” DIA. HD TURNTABLE/TRUCK HUB ASSY. & JOG SWITCH

- 240/480/60/3 ELECTRICAL REQUIREMENTS(PLEASE SPECIFY VOLTAGE)

- INTERMEDIATE SPRAY BAR W/ NOZZLES

- ½”PIN/KNUCKLE HINGED SEALESS DOOR W/ HANDLE, LATCH, AND SAFETY SWITCH

- 10 GAUGE TANK/12 GAUGE DOOR & CABINET STEEL PLATE MATERIAL W/ PRECAUTION BLUE PAINT

- 106” OVERALL HEIGHT

- 66”W X 72”D FLOOR SPACE REQUIRED W/ DOOR CLOSED

- 3800 LBS. SHIPPING WT.